DIVISION 11

SECTION 11 40 00

FOODSERVICE EQUIPMENT SPECIFICATIONS

| FOODSERVICE EQUIPMENT BIDDERS GUIDELINES |  |  |  |  |  | PAGE | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | EQUIPMENT BID QUOTE |  |  |  |  | PAGE | 5 |
|  | BID QUOTATION SUMMARY |  |  |  |  | PAGE | 6 |
| FOODSERVICE EQUIPMENT GENERAL CONDITIONS |  |  |  |  |  |  |  |
|  | ARTICLE 1 | - |  | CONTRACT DOCUMENTS |  | PAGE | 7 |
|  | ARTICLE 2 | - |  | DESIGNER |  | PAGE | 8 |
|  | ARTICLE 3 | - |  | OWNER |  | PAGE | 8 |
|  | ARTICLE 4 | - |  | KITCHEN EQUIPMENT CONTRACTOR |  | PAGE | 9 |
|  | ARTICLE 5 | - |  | SUB CONTRACTORS |  | PAGE | 12 |
|  | ARTICLE 6 | - |  | MISCELLANEOUS PROVISIONS |  | PAGE | 13 |
|  | ARTICLE 7 | - |  | TIME |  | PAGE | 13 |
|  | ARTICLE 8 | - |  | PAYMENTS AND COMPLETION |  | PAGE | 14 |
|  | ARTICLE 9 | - |  | INSURANCE |  | PAGE | 14 |
|  | ARTICLE 10 | - |  | CHANGES IN THE WORK |  | PAGE | 15 |
|  | ARTICLE 11 | - |  | PUBLICITY |  | PAGE | 15 |
| FOODSERVICE EQUIPMENT SPECIFIC CONDITIONS |  |  |  |  |  |  |  |
|  | PART I | - |  | GENERAL |  | PAGE | 15 |
|  | PART II | - |  | PRODUCTS |  | PAGE | 22 |
|  | PART III | - |  | EXECUTION |  | PAGE | 26 |
|  | PART IV | - |  | EQUIPMENT |  | PAGE | 27 |
| FOODSERVICE EQUIPMENT SCHEDULE |  |  |  |  |  | PAGE | 27 |
| FOOD SERVICE ITEMIZED EQUIPMENT |  |  |  |  |  | PAGE | 28 |

FOODSERVICE EQUIPMENT BIDDER’S GUIDELINES

      - Executed Bids shall be addressed and delivered as follows:

One copy to:

One copy to:	Clevenger Associates

Attn:  Brent Hall

11803 101st Avenue Court East, Suite 203

Puyallup, WA 98373

Delivery Time:

Method of Delivery:	By hand or mail at the responsibility of the bidder,

Enclosed in a sealed envelope; identified by name of Bidder and bearing the following project name:

PROJECT: 	HILTON HOME2 SUITES

VERSION 2.2

      - Preparation and Presentation of Bids

        - Bids shall be presented on enclosed forms.  Bids on forms other than the forms furnished will not be accepted.

        - Bids are to include price for each item, with a separate subtotal price for buyout equipment, fabricated equipment, delivery, installation and performance bond.  Any and all City, State, occupational and government taxes which are applicable to this installation shall be included in a grand total contract price.

        - Bidding firms name must be typed on each itemized Equipment Bid form and Bid Quotation Summary form in spaces provided.

        - Bid is to state any deviation or exceptions to any item and to acknowledge any addenda or change orders.  Submit, as part of bid, a list of sub-contractors and their qualifications as set forth in Article 5 of the Foodservice Equipment General Conditions.

      - Performance and Payment Bond

        - Bidders are required to show the total cost to Owner of the bonds set forth in Article 6 of the Foodservice Equipment General Conditions.  These costs are to be shown as a separate figure.  The cost of the bond will be deleted from the total price if the Owner chooses not to require a bond.

      - Commencement and Completion of Work

        - Equipment shall be installed in accordance with the dates listed below.  The act of submitting a bid for the work of this project implies agreement by the Bidder to conform to this schedule.

START OF APPROXIMATE INSTALLATION:

COMPLETION:

      - Drawings

        - The drawings, which constitute a part of the contract, indicate the arrangement and location of equipment.  Should it be necessary to deviate from this arrangement in order to meet conditions, such deviation shall be made without expense to Owner.

        - The data given on the drawings is reasonably exact, but extreme accuracy is not guaranteed.  Drawings are for the assistance and guidance of the Bidder and exact locations, distances and levels will be governed by the building.  The Bidder shall accept the contract with this understanding.

        - The following list of drawings as prepared by Clevenger Associates shall be considered as part of the bidding documents and shall bear an issue date of May 2016.

| FS-1 |  | EQUIPMENT PLAN AND SCHEDULE |
| --- | --- | --- |
| FS-2 |  | ELECTRICAL ROUGH-IN PLAN |
| FS-3 |  | PLUMBING ROUGH-IN PLAN |
| FS-4 |  | BUILDING WORKS PLAN |
| FS-5 |  | ELEVATIONS AND DETAILS |

      - Review of Contract Documents

        - Any conflict in these documents with Division 01 provisions, (the Architect’s General Conditions) the information or requirements in Division 01 will prevail.

        - It is the responsibility of the Bidder to carefully review the Contract Documents and shall at once upon discovery during preparation of bid; report to Designer any errors, ambiguities, inconsistencies or omissions at which time Designer will issue an addendum to all bidders.  Unless expressly stipulated, and in a timely manner, no additional allowances will be made for Contractors, or Manufacturers, for errors, omissions or ambiguities not reported at time of bidding.

      - Consultants Drawings

        - Consultant Drawings are not intended for construction purposes, but are information intended only for use by the Architect and Engineers as an aid in the design of the building and utility distribution systems and for bidding equipment purchase.  Consultant drawings in electronic format will not be issued by the Architect or Owner to third parties, including equipment suppliers, without express written consent  of the Consultant.

        - Consultant base Equipment Plans and Equipment Elevation Sheets will be provided to contracted equipment supplier in electronic format on request without charge.  Utility rough-in/connection schedules and plans will be provided in electronic format on payment by the KEC to Consultant of a nominal fee of $250 per drawing sheet for third party formatting.  Regardless of fees charged the transfer of drawings is not to be considered a sale and the Consultant makes no warranties, express or implied, of Merchantability or of fitness for a particular purpose.

        - Regardless of drawing formats provided it will remain the responsibility of equipment supplier to develop submittals in accordance with the Foodservice Equipment Specific Conditions and assume all required responsibilities there to.  Time spent in checking KEC re-submittals, for KEC deficient initial submissions, will be back-charged to the KEC at their standard billing rates.  Time spent in extraordinary coordination efforts by the Consultant necessitated by poor performance of the KEC will be back-charged to the KEC at their standard billing rates.  The Consultant is not to be liable for errors or omissions by the KEC’s use of electronic data provided by the Consultant or the development of data used in the submittal approval process.

      - Alternates and Substitutions

        - Bids are to include price for each item, with a separate subtotal price for buyout equipment, fabricated equipment, delivery, and installation and performance bond.  Bidders may submit, as an alternate, other brands and models of equipment.  Bids will not be considered if they do not include pricing for all base items, even if alternates are included.  Alternates shall be quoted on a separate page attached to the primary bid.

        - Unspecified alternates must be equal in all respects to the base equipment specified including all standard features.  Bids for such alternates must state the manufacturer, model number and include illustrations, specifications, capacities and operational data with the bid.

        - If alternates require different utility/building conditions, electrical, plumbing, ventilation, etc., from those specified, a complete list of those changes for each item shall be included with the alternate bid.  The cost of these changes may become the responsibility of the Kitchen Equipment Contractor.

        - All fabricated equipment shall be by one manufacturer acceptable to the Owner and Designer.  If the methods specified and detailed are not in accordance with the Kitchen Equipment Contractor’s methods he may quote his methods and standards as an alternate.  The alternate shall itemize all differences.

        - Alternates submitted after bidding will not be considered.  Acceptance or rejection of the alternates will be at the discretion of the Owner and/or Designer.

        - Alternate manufacturers/model numbers listed in itemized equipment specifications can be assumed by bidders to be acceptable substitutions and do not need to be identified in bid.

      - Consideration of Bids

        - The Owner reserves the right to waive irregularities and defects and to reject any and all proposals.  The Owner also reserves the right to accept a proposal which is deemed by the Designer to be in the best interest of the Owner.

      - Return of Documents

        - To receive future consideration, all drawings and specifications issued to Bidders must be returned to Clevenger Associates upon written request.

      - Foodservice Equipment Bid Quote

FIRM NAME:

| ITEM NO. | QTY. | DESCRIPTION | BID QUOTE |
| --- | --- | --- | --- |
| 1 | 1 | DISPLAY FREEZER |  |
| 2 | 1 | DISPLAY REFRIGERATOR |  |
| 3 | 1 | WARMING KETTLE |  |
| 4 | 1 | UNDERCOUNTER DISPLAY REFRIGERATOR |  |
| 5 | 1 | REFRIGERATED COLD WELL (DROP-IN) |  |
| 6 | 2 | MICROWAVE OVEN |  |
| 7 | 1 | WAFFLE IRON (OPERATOR FURNISHED) |  |
| 8 | - | SPARE NUMBER |  |
| 9 | 1 | JUICE DISPENSER (OPERATOR FURNISHED) |  |
| 10 | 1 | CONVEYOR TOASTER |  |
| 11 | 1 | BATTER MIX STARTER KIT(OPERATOR FURNISHED) |  |
| 12 | 1 | WAFFLE IRON (FUTURE) (OPERATOR FURNISHED) |  |
| 13 | 1 | HAND SINK WITH SOAP AND TOWEL DISPENSER |  |
| 14 | 1 | COFFEE MAKER (OPERATOR FURNISHED) |  |
| 15 | 1 | WALL SHELF (HIGH MOUNT) |  |
| 16 | 1 | WORK TABLE |  |
| 17 | 2 | WALL SHELF |  |
| 18 | 1 | MOBILE TABLE |  |
| 19 | 1 | MOBILE STORAGE SHELVING |  |
| 20 | - | SPARE NUMBER |  |
| 21 | - | SPARE NUMBER |  |
| 22 | - | SPARE NUMBER |  |
| 23 | 1 | MOBILE REACH-IN REFRIGERATOR |  |
| 24 | 1 | MOBILE REACH-IN FREEZER |  |
| 25 | 1 | UTENSIL RACK |  |
| 26 | 1 | DISH/POT WASHING SINKS |  |
| 27 | 3 | WALL SHELF (HIGH MOUNT) |  |
| 28 | 1 | PRE-RINSE WITHFAUCET |  |
| 29 | 1 | RACK SHELF (WALL MOUNT) |  |
| 30 | 1 | UNDERCOUNTER DISHMACHINE (HIGH TEMP) |  |

BID QUOTATION SUMMARY

FIRM NAME:

SUBTOTAL BUYOUT EQUIPMENT

SUBTOTAL FABRICATED EQUIPMENT

TAXES

DELIVERY AND INSTALLATION

GRAND TOTAL

PERFORMANCE BOND

DEMOLITION, REMOVAL, STORING AND

CLEANING OF EXISTING EQUIPMENT

LIST OF SUB-CONTRACTORS

WE ACKNOWLEDGE ADDENDUM(S) ______________________, DATED

THE ABOVE BID IS IN ACCORDANCE WITH THE BID DOCUMENTS, EXCEPT AS NOTED.

THE UNDER SIGNED ACKNOWLEDGES THAT THEY HAVE READ AND UNDERSTAND THE INSTRUCTIONS TO BIDDERS, GENERAL CONDITIONS AND SPECIFIC CONDITIONS AND, IF THE SUCCESSFUL BIDDER, WILL FULLY COMPLY WITH ALL ARTICLES AND SECTIONS CONTAINED THEREIN.

FIRM

SIGNATURE

PRINT NAME

DATE

FOODSERVICE EQUIPMENT GENERAL CONDITIONS

- CONTRACT DOCUMENTS

      - DEFINITIONS

        - The Contract Documents

          - Contract Documents shall consist of:	The specification dated May 2016  which consist of Foodservice Equipment Bidder’s Guidelines, Foodservice Equipment General Conditions, Foodservice Equipment Specific Conditions, Foodservice Equipment Schedule, Foodservice Itemized Specifications, Foodservice Equipment Details; the Drawings dated May 2016; add addenda issued prior to the execution of the contract; and all modifications thereto.

        - Contract

          - The Contract Documents form the Contract. The Contract represents the entire and integrated agreement between the parties hereto and supersedes all prior negotiations; representations or agreements whether written or oral, including the bidding documents.

        - The Work

          - The term Work includes all labor, material and services necessary to produce and install the equipment required by the Contract Documents.

      - EXECUTION, CORRELATION AND INTENT

        - The Drawings and the Specifications are intended to be complimentary so that any work exhibited in the drawings is not mentioned in the specifications, or vice-versa, and is to be executed the same as if both mentioned in the specifications and exhibited in the drawings to the true intent and meaning of the said drawings and specifications when taken together.  Large scale and full-sized drawings shall be followed in preference to small scale drawings.  Figured dimensions shall be followed in preference to scale measurements.

        - Titles and headings to sections in these Contract Documents are introduced for convenience and shall not be taken as a correct or complete segregation of several units of materials and labor.  No responsibility either direct or implied will be assumed by the Owner or the Designer for errors or omissions by the Kitchen Equipment Contract, due to real or alleged error in arrangement or content of matter in the Contract Documents.

        - The drawings and specifications are advisory and for information purposes only.  They are not intended to be, and shall not be used for construction purposes.  They are to be used by licensed Architects and Engineers for preparing their stamped and sealed documents and for the Kitchen Equipment Contractor for preparing dimensioned rough-in drawings, brochure submittals and shop drawings.

- DESIGNER

      - DEFINITION

        - The Designer shall be known as Clevenger Group d.b.a. Clevenger Associates, 11803 101st Avenue Court East, Suite 203, Puyallup, Washington 98373, and identified as such in the Agreement and is referred to throughout the Contract Documents as if singular in number and masculine in gender.  The term Designer means Designer or his authorized representative.

      - DUTIES AND RESPONSIBILITIES

        - Nothing contained in the Contract Documents shall create any contractual relationship between the Designer and the Kitchen Equipment Contractor.

        - The Designer may be the Owner’s representative during construction and until final payment unless otherwise specified in the Foodservice Equipment Specific Conditions.  Designer will have authority to act on behalf of Owner to the extent provided in the Contract Documents, unless otherwise modified by written instrument which will be shown to Kitchen Equipment Contractor.  Designer may advise and consult with Owner, and all of Owner’s instructions to Kitchen Equipment Contractor shall be issued through the Designer or Owner’s representative.

        - Claims, disputes and other matters in question between Kitchen Equipment Contractor and Owner relating to execution or progress of the work or interpretation of the Contract Documents may be referred initially to designer for a decision which he will render in writing within a reasonable period of time.

        - All interpretations and decisions of the Designer shall be consistent with the intent of the Contract Documents.  In his capacity as interpreter and judge, he will exercise his best efforts to insure faithful compliance by both Owner and Kitchen Equipment Contractor.

        - Checking rough-in drawings, shop drawings, details and equipment drawings by Designer is for design concept only, and does not relive the Kitchen Equipment Contractor of responsibility for compliance with design drawings, details and specifications, verification of utilities with equipment requirements for conformity and location, and verification of all dimensions of equipment and building conditions or reasonable adjustments due to deviations.

- OWNER

- DEFINITION

  - Owner is the person or organization identified as such in the Agreement and is referred to throughout the Contract Documents as if singular in number and masculine in gender.  The term Owner means the Owner or his authorized representative.

- ACCESS, AUTHORITY AND STOPPAGE

  - The work shall be available for inspection at any time by a representative of the Owner.

  - All rejected work or material shall be immediately replaced.

  - Such stoppage, suspension or forfeiture shall not in any way invalidate any terms of the Contract, and no extra charge will be allowed the Kitchen Equipment Contractor by reason of such stoppage or suspension.

- WORK UNDER PROTEST

  - If the Owner orders certain work done, the Kitchen Equipment Contractor shall comply promptly and fully with such an order but in so doing shall not forfeit any right which he may have under this Contract to claim extra payments.  Any claim which the Kitchen Equipment Contractor desires to make regarding work so ordered shall be made to Owner in writing ten (10) days of receipt of work order.

- OWNER’S RIGHT TO DO WORK

  - If Kitchen Equipment Contractor should neglect to perform the work properly or any provision of this contract, the Owner, after three (3) days written notice to the Kitchen Equipment Contractor and his surety, may correct deficiencies and deduct the expense from payment due the Kitchen Equipment Contractor.

- EMERGENCY REPAIRS

  - Owner reserves the right at all times to make emergency repairs without voiding or impairing guarantee or relieving Kitchen Equipment Contractor of responsibility during guarantee period.  Any piece of equipment producing objectionable noise must be immediately repaired, removed or replaced.

- Owner reserves the right to let other contracts in connection with this work, and the Kitchen Equipment Contractor shall afford these other Contractor’s reasonable opportunity for the introduction and execution of the work, and shall properly connect and coordinate this work with theirs, as required.

- KITCHEN EQUIPMENT CONTRACTOR

- DEFINITION

  - The Kitchen Equipment Contractor (KEC) is the person or organization identified as such in the agreement and is referred to throughout the Contract Documents as if singular in number and masculine in gender.  The term Kitchen Equipment Contractor means the Kitchen Equipment Contractor or his authorized representative.

- REVIEW OF CONTRACT DOCUMENTS

  - Kitchen Equipment Contractor shall carefully compare Contract Documents and shall at once report to Owner any errors, ambiguities, inconsistencies or omissions he may discover.  Unless expressly stipulated, and in a timely manner, Kitchen Equipment Contractor shall be liable to Owner or Designer for any damage resulting from any such errors, inconsistencies or omissions in the Contract Documents.  Kitchen Equipment Contractor shall not do any work without requesting Drawings, Specifications or Modifications and receiving prior written authorization from Owner or Designer.

- SUPERVISION, FABRICATION, INSTALLATION AND PROCEDURES

  - Kitchen Equipment Contractor shall supervise and direct the work, using his best skill and attention.  He shall be solely responsible for all construction means, methods, techniques and procedures for coordinating all portions of work under the Contract and for coordinating the installation with the General Contractor, so as not to interfere with or delay the overall construction of the project.

  - During the entire installation period, the Kitchen Equipment Contractor shall provide a competent foreman for erection of the equipment regarding connections and installations.  The Designer and Owner shall be sole judges as to the competence of the said supervisor.

- LABOR, MATERIAL AND TRADE UNIONS

  - Unless otherwise specifically noted, Kitchen Equipment Contractor shall provide and pay for all labor, materials, equipment, tools, transportation and other facilities and services necessary for proper execution and completion of the Work.  The Kitchen Equipment Contractor shall at all times enforce strict discipline and good order among his employees and shall not employ anyone not skilled in the task assigned to him or who will create disharmony on the site.

  - All work shall be performed in connection with this contract shall be by workmen of firms whose employees are not objectionable to the various trade unions under construction of the premises.  The Kitchen Equipment Contractor shall be wholly responsible for all trade union relations and Owner shall not be liable in any way for delays or claims arising through such causes.

- WARRANTY

  - All equipment, fixtures and materials furnished and installed shall be guaranteed against defects in workmanship and material; and all repairs and replacements which may have become apparent and necessary by reason of such defects, during the first year after final completion and acceptance of equipment installation, shall be made by the Kitchen Equipment Contractor at his own cost and expense without charge to the Owner.  All such repairs and replacements shall be made at a time and during hours satisfactory to the Owner.

  - For all commercially manufactured equipment that have refrigeration systems, Kitchen Equipment Contractor to furnish additional four (4) year warranty on all compressors.

  - Warranty period shall commence with final acceptance of installation by Owner.

  - Components of equipment subject to replacement prior to one-year use and those items which may bail due to improper or inadequate periodic maintenance by the Owner/Operator are not intended to be included within the scope of warranty.

- TAXES AND DUTY

  - Kitchen Equipment Contractor shall pay all duties, sales, consumer, use and other similar taxes, license and permits as related to his work and required by law.

- PERMITS AND CERTIFICATES

  - Kitchen Equipment Contractor shall obtain and pay for all necessary permits, inspections, certificates and licenses require and necessary for performance of the work and post all notices required by law and comply with all laws, ordinances and regulations bearing on conduct of the work as drawn and specified.

  - Procure all necessary certificates of acceptance or completion required and issued by state, municipal or other authorities and deliver these to Owner.  Owner may withhold any payments which may become due the Kitchen Equipment Contractor until the necessary certificates are delivered to him.

- SERVICE REPRESENTATIVES

  - Provide a competent service representative to be present when installation is put into operation.  He shall lubricate and put into proper operation all equipment and instruct Owner’s employees in proper use and maintenance of all items in this contract.

- SHOP DRAWINGS, PRODUCT DATA AND SAMPLES

  - After award of contract and before proceeding with fabrication of any item, the Kitchen Equipment Contractor shall submit for checking, drawings produced by the Kitchen Equipment Contractor from verified dimensions and conditions in compliance with requirements of the Foodservice Equipment Specific Conditions.

  - After award of contract and before proceeding with the purchase of manufactured equipment, the Kitchen Equipment Contractor shall submit product data in compliance with requirements of the Foodservice Equipment Specific Conditions.

  - Submit samples of materials and parts of work requested by the Designer for adequately judging character and quality of work.

  - The checking of such drawings, product data and samples shall not relieve the Kitchen Equipment Contractor from responsibility of any deviation from the drawings and specification unless such deviations are approved in writing by the Owner or the Designer.

- ACCESS AND USE OF SITE

  - No provisions shall be made for receipt of storage of any items delivered to job site before commencement of installation except as negotiated with the Owner.  All items must be received and accounted for by the Kitchen Equipment Contractor until acceptance by the Owner or his Agent.

  - No signs and nameplates of any type shall be allowed to be displayed on any part of this work or on or about the Owner’s premises unless authorized in writing by the Owner or Designer.

- CUTTING, FITTING AND PATCHING OF WORK

  - Do all cutting and fitting on equipment for other sub-contractors to make their work fit, or as Owner or Designer may direct.  Should any repairs be required due to neglect by other contractors, repairs must be approved by Owner or Designer before the work is performed.  All extra charges must be approved and all repairs must be noted in writing before work is preformed, stipulating price and by whom extra expense shall be paid.  In case the Kitchen Equipment Contractor does not secure an extra, expenses shall be borne by him.  No cutting, drilling or altering of any kind shall be done to building by the Kitchen Equipment Contractor without first obtaining permission from the Architectural Representation.

  - The Kitchen Equipment Contractor shall be responsible for any patching of ceiling, floor or wall surfaces required by reason of his work and caused by negligence of his employees.  Patches of any kind required will be repaired and charge by the Owner/General Contractor to the party causing damage.

- CLEAN-UP

  - The Kitchen Equipment Contractor shall keep his area of responsibility clean and free from debris and shall leave his area in a finish condition.  All cartons, crates and surplus material must be cleaned up daily and removed from site at the Kitchen Equipment Contractor’s expense.  If the Kitchen Equipment Contractor is not diligent in the clean up, the Owner reserves the right to clean up his material and back-charge his contract.

- ROYALTIES AND PATENTS

  - Kitchen Equipment Contractor shall pay all royalties and license fees.

  - Kitchen Equipment Contractor hereby covenants and agrees to save Owner and Designer harmless and indemnify the Owner and Designer from payments of any royalties, damages, losses or expense for suits, materials and methods used in fabrication of this equipment.

- SUBCONTRACTORS

- DEFINITIONS

  - A sub-contractor is a person or organization who has a direct contract with the Kitchen Equipment Contractor to perform any of the work at the site.  The term Sub-Contractor is referred to in the Contract Document as singular in number and masculine in gender.  The term Sub-Contractor means the Sub-Contractor or his authorized representative.

- AWARDS OF SUB-CONTRACTS

  - Names and addresses of all Sub-Contractors shall be furnished to the Owner and Designer, and selection of sub-contractors must be approved by them.  If, in their judgment, any fail to perform the work in strict accordance with the drawings and specifications, the Kitchen Equipment Contractor, after due notice from the Owner or Designer, shall discharge same, but this shall in no way release the Kitchen Equipment Contractor from his obligations and responsibility  under the Contract.

- SUB-CONTRACTUAL

  - Every sub-contractor shall be bound by terms and provisions of the Contract Documents as far as applicable to his work.  Nothing contained herein shall create any contractual relation between any Sub-Contractor and Owner.

  - Kitchen Equipment Contractor shall be fully responsible to Owner for acts and omissions of his sub-contractors.

- MISCELLANEOUS PROVISIONS

- LAWS AND ORDINANCES

  - All work and materials shall comply with all state and federal laws, municipal ordinances, regulations and directions of inspectors appointed by proper authorities have jurisdiction.  In the event any of the conditions of these specifications violate the code for any industry, then such code conditions shall prevail.  The Kitchen Equipment Contractor shall state in his bid where the code is at variance with these specifications.

- ASSIGNMENT OF CONTRACT

  - The Kitchen Equipment Contractor shall not assign this contract or any part hereof without written consent of the Owner.  Owner may assign its right hereunder to an assignee or its affiliate subject to such affiliate’s assumptions of the Owner’s obligations.

- CLAIMS FOR DAMAGES

  - Should either party to the contract suffer injury or damage to person or property because of any act or omission of the other party or any of his employees, agents or other for whose acts he is legally liable, claims shall be made in writing to such other party within a reasonable time after first observance of such injury or damage.

- PERFORMANCE BOND AND LABOR MATERIAL

  - Owner shall have the right to require Kitchen Equipment Contractor to furnish a performance and payment bond in a form acceptable to the Owner or the amount of the contract, covering faithful performance of the contract and payment of all obligations arising there under.

- TESTS

  - If Contract Documents, laws ordinances, rules and regulations or orders of any authority having jurisdiction require any work to be inspected, tested or approved, the Kitchen Contractor shall give Designer timely notice of its readiness and of date arranged so Designer may observe such inspection, testing or checking.  The Kitchen Equipment Contractor shall bear all costs of such inspections, tests and checking’s unless otherwise provided.

- TIME

- PROGRESSION AND COMPLETION

  - Time is of the essence in this agreement and acceptance constitutes a guarantee that the Kitchen Equipment Contractor can and will obtain all materials, equipment, and manpower, upon notice to proceed, schedule in accordance with the General Contractor’s program to permit overall completion of the entire building project on schedule. The Kitchen Equipment Contractor shall coordinate his work with the progress schedule, as prepared and updated periodically by the General Contractor.

- PAYMENTS AND COMPLETION

- CLAIMS FOR PAYMENT

  - Claims for payment shall be submitted by the Kitchen Equipment Contractor to Owner as stipulated in the Foodservice Equipment Bidder’s Guidelines and/or the Owner-Contractor Agreement (Contract).

  - The Kitchen Equipment Contractor to propose his terms required for payment.  Proposed terms subject to contract negotiations.

- SUBSTANTIAL AND FINAL COMPLETION

  - The Kitchen Equipment Contractor shall give written notice to Owner that the work is completed and shall submit evidence satisfactory to Owner in the form of receipted bills for all materials and equipment furnished, waiver of liens from sub-contractors and other forms a may be required by the Owner to insure that all payrolls, material bills and other indebtedness connected with the Work have been paid.

- PAYMENTS TO SUBCONTRACTORS

  - The Kitchen Equipment Contractor shall pay each sub-contractor, upon receipt of payment from Owner, an amount equal to percentage of completion allowed to the Kitchen Equipment Contractor on account of such sub-contractor’s work, less the percentage retained from payment to the Kitchen Equipment Contractor.  The Kitchen Equipment Contractor shall also require each sub-contractor to make similar payments to his sub-contractors.  The Kitchen Equipment Contractor, at Owner’s request, shall furnish proof of payment of obligations to such contractors or suppliers.  If the Kitchen Equipment Contractor fails to discharge such obligations, the Owner, upon written notice to the Kitchen Equipment Contractor, shall withhold an amount from the Kitchen Equipment Contractor’s payment deemed to be reasonably sufficient to pay any claim.

- INSURANCE

- PUBLIC LIABILITY,	CASUALTY AND WORKER’S COMPENSATION INSURANCE

  - Certificates of liability, casualty and Workman’s Compensation insurance shall be filed with Owner when required and will be subject to his checking both as to amount and adequacy of their protection.

  - Make all payments in accordance with unemployment, old age and other insurance and social security of various governments, pursuant to law for that purpose made and provided, whether enacted at the time or prior to execution of contract or during progress of work hereunder, and assume all liability for compliance with requirements thereof.

  - Assume all liability for injuries to or loss of any Owner’s property or the property of any other contractor which may be employed by Owner on said premises, or to any adjoining property or property of any third person which may be caused directly or indirectly by the Kitchen Equipment Contractor under this covenant is absolute and is not dependent upon any questions or negligence of his, his subcontractors, to their employees, and failures of Owner or Designer to direct the Kitchen Equipment Contractor to take any particular precaution or refrain from doing any particular act will not excuse the Kitchen Equipment Contractor in case of such damage.

- FIRE INSURANCE

  - Owner will insure building or other work included in this contract against loss or damage by fire and against loss and damage covered by standard extended coverage insurance endorsements, the amount of insurance at all times at least equal to amount paid on account of work or materials, and plus the value of work or materials furnished or delivered to the job site but not yet paid for by Owner.  The policies shall be in the names of the Owner and the Kitchen Equipment Contractor, as their interests may appear.

- CHANGES IN THE WORK

- CHANGES

  - No changes shall be made, no bill for damages, alterations, modifications, deviations and extra orders be recognized or paid for except upon written order of the Owner.

  - No payment of overtime shall be allowed except upon written order of Owner.

  - Materials of equal quality and merit of variations in construction methods from those detailed and specified may be used, but only upon written consent and checking of designer.

- PUBLICITY

- USE OF PUBLICITY

  - Any publicity giving reference to this project, whether in the form of press releases, brochures, photographic coverage or verbal announcement, shall be only with the general or specific permission of the Owner, and in all instances shall give due mention of the Designer.

FOODSERVICE EQUIPMENT SPECIFIC CONDITIONS

  - GENERAL

      - RELATED DOCUMENTS

        - General provisions of the contract, including Foodservice Equipment General Conditions, Supplementary Conditions and General Documents, other Division 1 Specification Documents and other Division 1 specification sections apply under this section.

      - SCOPE OF WORK

        - Furnish all labor, materials and services necessary for the procurement and installation of foodservice equipment in strict accordance with the Contract Documents and local codes including that which is reasonably inferred.  No extra charge will be allowed for that which the Kitchen equipment contractor should be familiar.

        - Supervise and provide required instructions for work to be performed by other contractors in connect with requirements for all equipment under this section.

        - Related Sections include:

          - Division 1 Section "Project Management and Coordination" for submitting and distributing meeting and conference minutes.

          - Division 1 Section "Submittal Procedures" for submitting schedules and reports.

          - Division 1 Section "Quality Requirements" for submitting a schedule of tests and inspections.

      - RELATED WORK SPECIFIED ELSEWHERE

        - Floor and setting beds, quarry tile and base, masonry pads, walls and finishes, ceilings and related building work:  Divisions 03 through 09.

        - Wall backing to support all wall mounted equipment: Division 6 and 9.

        - All water, waste, indirect waste piping from sinks, services to the equipment including all shut-off valves, plumbing trim, traps, etc., and final connections to the equipment except as specified herein:  Division 22 and 23.

        - All floor sinks and floor drains:  Division 22.

        - Piping sleeves for drain lines through building floors:  Division22.

        - All electric services and components including wiring to and final connections to all equipment except as specified herein:  Division 26.

        - Ground type receptacles for all wall mounted outlet to be used for plug-in equipment:  Division 26.

      - OWNER/PURVEYOR FURNISHED EQUIPMENT

        - Obtain and coordinate manufacturer and model number not less than 60 days before equipment is required.

        - Obtain and coordinate utility requirements.

      - EXISTING EQUIPMENT – N/A

      - REGULATIONS

        - All work and materials shall be in accordance with the latest rules and/or regulations of agencies/authorities having jurisdiction.

        - All regulations, including building codes, and other codes applying to this jurisdiction should be followed.  In addition all equipment shall comply with the following:

          - Local Health Code.

          - National Electric Manufacturer’s Association (N.E.M.A.).

          - Underwriters Laboratories Inc., (U.L.), must bear label.  National Electric Code, (N.E.C.).

          - National Sanitation Foundation, (N.S.F.), including NSF-7, must bear label in jurisdictions requiring the same.

          - Occupational Safety and Health Act (O.S.H.A.) Standards.

          - Hazard Analysis Critical Control Path (H.A.C.C.P.) Standards.

          - American Disability Act (A.D.A.) Standards.

          - Federal Energy Independence and Security Act of 2007 (HR6).

        - The Contract Documents shall govern wherever they require larger sizes or higher standards than are required by regulations.

        - The regulations shall govern whenever the Contract Documents require something which will violate the regulations.

        - When seismic regulations are applicable, all equipment shall be fabricated and installed in accordance with those regulations.  All seismic requirements shall be shown on all submittals.  Submit requested information to the agencies and authorities having jurisdiction.

        - No extra charge will be paid for furnishing items required by the regulations, but not specified and/or shown on the drawings.

        - Rulings and interpretations of the enforcing agencies shall be considered a part of the regulations.

      - EQUIPMENT SUBSTITUTIONS

        - Refer to Division 01 – General Requirements.

        - Bids are to include price for each item, with a separate subtotal price for buy-out equipment, fabricated equipment, delivery, installation, and performance bond.  Bidders may submit, pre-approved substitutions, other brands, and models of equipment. Bids will not be considered if they do not include pricing for all base items, even if substitutions are included. Substitutions shall be quoted on a separate page attached to the primary bid.

        - Unspecified substitutions must be equal in all respects to the base equipment specified including all standard features.  Bids for such substitutions must state the manufacturer, model number and include illustrations, specifications, capacities and operational data with the bid.

        - If substitutions require different utility/building conditions, electrical, plumbing, ventilation, etc., from those specified, a complete list of those changes for each item shall be included with the substitute bid. The cost of these changes may become the responsibility of the Kitchen Equipment Contractor.

        - Substitutions shall be submitted for approval prior to bid date. Acceptance or rejection of the substitutions will be at the discretion of the Owner and/or Designer prior to the bid date.

        - Substitute manufacturer/model numbers listed in itemized equipment specifications can be assumed by bidders to be acceptable substitutions and do not need to be identified in bid.

      - REVIEW OF CONTRACT DOCUMENTS

        - Unless expressly stipulated, and in a timely manner, no additional allowances will be made for Contractors or Manufacturers for errors, omissions or ambiguities not reported at time of bidding.

        - Carefully review and compare the Contract Documents and at once report to Owner and/or Designer any errors, ambiguities, inconsistencies or omissions.  Unless expressly stipulated, and in a timely manner, Kitchen Equipment Contractor shall be liable to Owner or Designer for any damage resulting from such errors, inconsistencies or omissions in the Contract Documents.  Work shall not be done without approved Drawings, Specifications and/or Modifications and without receiving prior written authorization from Owner or Designer.

      - WARRANTY

        - All equipment, fixtures and materials furnished and installed shall be guaranteed against defect in workmanship and material.  All repairs and replacements which may have become apparent and necessary by reasons of such defects, during the first year after final completion and acceptance of equipment installation, shall be made without cost and expense to the Owner.  All such repairs and replacements shall be made at a time and during hours satisfactory to the Owner.

        - For all commercially manufactured equipment that has refrigeration systems and semi-hermetic compressors, furnish an additional four (4) year warranty on all compressors.

        - Warranty period shall commence with the date of final acceptance of installation by Owner.

        - Components of equipment subject to replacement prior to one(1) year’s use and those items which may fail due to improper or inadequate periodic maintenance by the Owner/Operator are not intended to be included within the scope of warranty.

        - Provide all labor, material, refrigerants, and incidental expenses to maintain the temperatures specified on all refrigeration systems.  Systems to be kept in first class working condition for a period of one (1) year from date of acceptance by Owner, or the date systems are put into operation, whichever occurs first, without additional cost to the Owner.

      - SUBMITTALS

        - Use of Consultant’s Drawings

          - Consultant drawings are not intended for construction purposes, but are information intended only for use by the Architect and Engineers as an aid in the design of the building and utility distribution systems and for bidding equipment purchase.  Consultant drawings in electronic format will not be issued by the Architect or Owner to third parties, including equipment suppliers, without express written consent of the Consultant.

          - Consultant base Equipment Plans and Equipment Elevation Sheets will be provided to contracted equipment supplier in electronic format on request without charge.  Utility rough-in/connection schedules and plans will be provided in electronic format on payment by the KEC to Consultant of a nominal fee of $250 per drawing sheet for third party formatting.  Regardless of fees charged the transfer of drawings is not to be considered a sale and the Consultant makes no warranties, express or implied, of Merchantability or of fitness for a particular purpose.

          - Regardless of drawing formats provided it will remain the responsibility of equipment supplier to develop submittals in accordance with the Foodservice Equipment Specific Conditions and assume all required responsibilities thereto.  Time spent in checking KEC re-submittals, for KEC deficient initial submissions, will be back-charged to the KEC at their standard billing rates.  Time spent in extraordinary coordination efforts by the Consultant necessitated by poor performance of the KEC will be back-charged to the KEC at their standard billing rates.  The Consultant is not to be liable for errors or omissions by the KEC’s use of electronic data provided by the Consultant or the development of data used in the submittal approval process.

        - Product Data

          - After award of contract and before proceeding with the purchase of manufactured equipment, submit a bound Electronic PDF file copy of specification sheets consisting of:

            - Hard Cover.

            - Title Sheet.

            - Index all items with columns for:  Item number, quantity, description and status (fabricated, manufactured, by Owner/Operator, by General Contractor, et.).

            - A typewritten lead sheet for each manufactured equipment item showing:  Item number, quantity, description; manufacturer’s name, address and telephone;  model number; optional finishes, equipment, accessories and modifications; utilities required and special notes.

            - Manufacturer’s specification sheets and/or drawings.

          - Submittals not in the above format will be returned for re-submittal.

        - Drawing Submittals

          - For all drawing submittals provide a bound Electronic PDF file copy.

          - Plan drawings shall be at a scale of ¼” – 1’-0” (1:50).

        - Rough-In Drawings

          - Drawings shall be dimensioned, showing ventilation requirements, floor and wall sleeves, plumbing, gas, steam, and electrical connections, including those items supplied by the Owner.  Provide concrete pad dimensions, depressions and special conditions as required for equipment.

          - The following shall each be drawn on separate sheets and/or plans: Itemized Equipment Plan/Schedule; Plumbing; Electrical; Building Works and Ventilation; Refrigeration and Beverage Systems.

          - Utilities shall be stubbed out of walls wherever possible.

          - Verify mechanical, electrical, and ventilation rough-in and sleeve/conduit locations before floor slabs are poured.

          - In the event rough-in has been accomplished before the award of the contract, check existing facility and furnish all equipment to suit building conditions and utilities.  No extra charges shall be allowed for utility changes to fit equipment during installation and connection.

        - Wall Backing Drawings

          - Prepare and submit wall backing drawings.  The drawings shall show the location and size of all wall backing required.  The drawings shall be submitted for checking and to the General Contractor in time for the wall backing to be installed prior to closing of the walls.

        - Shop Drawings

          - Prepare and submit shop drawings for all special fabricated items of work included in this contract.  The detail drawings shall be submitted at minimum of ¾” (1:20) scale for elevations and 1-1/2” (1:10) scale for sections.  Drawings shall show all dimensions, all details of construction, installation and relation to adjoining and related work.  Drawings shall show all reinforcements, anchorage and other related work required for the complete installation of all fixtures.

        - Record Drawings

          - At the conclusion of the project and prior to final payment, provide updated record Drawings incorporating all changes that occurred during construction in the form of CAD disks and one (1) hard copy set.

            - Submittal drawings shall include as applicable; mep rough-ins, custom fabrication, engineered systems including exhaust hood, refrigeration, etc.

        - Checking

          - Checking product data, rough-in drawings, wall backing drawings, shop drawings, and refrigeration drawings by Designer is for design concept only, and does not relieve the Kitchen Equipment Contractor of responsibility for compliance with Contract Documents, verification of utilities with equipment requirements for conformity and location, verification of all dimensions of equipment and building conditions or reasonable adjustments due to deviations.

          - Drawings shall be prepared on the Kitchen Equipment Contractor’s sheets and by his employees.  Drawings and any part thereof created by photograph, paste-up, or other methods using Designer’s drawings and/or details will be returned for re-submittal.

          - Submittals and checking shall be accomplished before ordering equipment or starting fabrication.

        - Requests for Information (RFI’s)

          - All RFI’s to be submitted per General Conditions or otherwise by e-mail to info@clevengerassoc.com or if known, by e-mail directly to the project manager.

        - Mailing and Distribution

          - All drawings shall be delivered via E-mail, FTP site or General Contractor share site.

          - After checking, supply the specified number of distribution prints and as many as ten (10) corrected product data books as directed by the Architect, General Contractor or Owner.

      - PARTS AND SERVICE MANUALS

        - Furnish two (2) bound sets of parts and service manuals.

          - The manuals shall include a source directory for parts and service for all items.

          - The manuals shall be submitted in time to allow review and transmittal to the Owner/Operator prior to start-up and demonstration of the equipment.  Manuals must be submitted before the Owner will issue final acceptance of the installation.

            - Provide a bound electronic pdf file copy on cd of the record drawings as part of this submittal.

      - VERIFICATION AND COORDINATION OF PROJECT/DATA

        - Range Lines – N/A

        - Pans and Inserts

          - Verify sizes with Owner on the following items before ordering or fabrication:

            - Steam Pans.

            - Sheet Pans.

            - Trays.

            - Glass and Cup Racks.

        - Quietness of Operation

Quietness of operation of all food service and refrigeration equipment is a requirement.  Remove or repair any equipment producing abnormal and objectionable noises.

        - Delivery and Entry

Verify all conditions at the building, particularly door openings and passageways for large equipment.  Coordinate with the General Contractor access to insure delivery of equipment to the required areas.  Coordination shall include, but not limited to, early delivery, hoisting, window removal and/or delay of wall construction.  All special equipment, handling charges, window removal, etc. shall be paid for by the Kitchen Equipment Contractor.

        - Connection Terminals

All equipment will be complete with standard connections as they relate to their Country of Origin.  It shall be the responsibility of the Kitchen Equipment Contractor to provide any and all required adaptors to assure the proper connection to the conditions at the job site.

        - Site Verification

Notify Designer, Owner and the General Contractor in writing if, in the Kitchen Equipment Contractor’s opinion, the jobsite is not adequate to insure proper installation of the equipment.  Notification shall be in writing with sufficient time to effect corrective measures to meet the installation schedule.

- PRODUCTS

- COMMERCIALLY MANUFACTURED EQUIPMENT

- All items of standard equipment shall be the latest model at time of delivery.

- Follow manufacturer’s directions used to fulfill this contract which cover points not necessarily shown on the drawings or specifications.

- All doors shall be hinged as shown on plans.

- PLUMBING WORK

- Provide suitable pipe slots, chases and/or do all drilling, punching and cutting of equipment required to provide access for Division 22 connections and/or runs.  Such work performed at the job site shall be of the same quality as similar work in the shop.

- To insure proper clearance for cleaning, all horizontal piping lines shall be run at the highest possible elevation through equipment and not less than 6” (150 mm) above floor, wherever possible.

- Indirect waste piping shall be installed in accordance with the local codes.  Piping shall run as described hereinafter, and shall discharge into floor sinks.  Extend piping to a point of at least 2” (50 mm) above rim of floor sink and cut bottom on 45o angle.  All indirect waste piping shall be installed and routed in a manner to insure proper drainage and shall conform with shelves, spaces, equipment or building conditions.  Indirect waste piping to be secured to fixture.

  - Indirect waste piping form ice bins, ice pans or similar items shall be insulated to prevent condensation.

- Water inlets shall be located above the positive water level to prevent siphoning of liquids into the water system.  Wherever conditions shall require a submerged inlet.  Provide a suitable type of check valve and vacuum breaker.

- Where exposed, piping and fittings shall be chrome plated.

- All faucets intended to dispense water for human consumption shall be manufactured from pure stainless steel that contains zero lead, no brass allowed in the waterways of the product. Finish shall be polished stainless steel

| Type 1: | Faucet: ½” inlets 8” adjustable wall mount base; 12” swing nozzle; elbows and supply lines. |
| --- | --- |
|  | TANDS Model # B-0231-230KIT |
| Type 2: | Faucet: ¾” inlets, 8” adjustable wall mount base, 14” swivel nozzle. |
|  | TANDS Model # B-0278 |
| Type 3: | Faucet: ½” inlets, single mount dual control, deck mount base, 6” swing nozzle. |
|  | TANDS Model # B-0202 |
| Type 4: | Faucet ¾” inlets, 8” adjustable deck mount base, 14” swing nozzle. |
|  | TANDS Model # B-0293-14 |
| Type 5: | Faucet: ½” inlets, single hole dual control, deck mount base, 5-1/2” rigid gooseneck. |
|  | TANDS Model # B-0300 |
| Type 6: | Faucet: ½” inlets, single hole single control, deck mount base, 6” swig nozzle. |
|  | TANDS Model # B-0207-CR |
| Type 7: | Pre-Rinse Unit: Easy install, ½” inlets, 8” adjustable wall mount base, 6” wall bracket, vacuum breaker, elbows and supply lines. |
|  | TANDS Model # B-2278-EL |
| Type 8: | Pot filler: ½” inlet, single mount single control, wall mount base, 18” double joint swivel nozzle. |
|  | TANDS Model # B-0592 |
| Type 9: | Dipperwell Faucet: dipperwell faucet with drain, stainless steel bowl and inner overflow cup, ½” inlet. |
|  | TANDS Model # B-2282-01 |
| Type 10: | Water Station with pedestal type glass filler, stainless steel drip pan, ¼” tailpiece for copper tubing, 1-1/4” drain. |
|  | TANDS Model # B-1230-12 |
| Type 11: | Pre-Rinse Unit: Easy install, ½” inlets, 8” adjustable wall mount base with elbows, add-on faucet with 12” swing spout, 6” wall bracket, vacuum breaker. |
|  | TANDS Model # B-2278-A12-El |
| Type 12: | Glass Rack Fill Faucet: pre-rinse unit; Easy install, single deck dual control, 6” wall bracket, vacuum breaker, B-0107-A spray valve, 18” flexible supply hoses. |
|  | TANDS Model # B-0113-BVB-107A |

- DRAINS AND WASTES

1.	Furnish all necessary drains and wastes with the equipment and as follows:

| Type 1: | Drain: 1-1/2” AND 2” NPT waste valve with removable snap-in strainer. Rotary waste valve with twist handle 3-1/2” sink opening, 2” NPT male outlet AND 1-1/2” NPT male adapter. TANDS Model # B-3950 |
| --- | --- |
| Type 2: | Basket Drain: 1.5 id. Basket strainer; polished chrome. TANDS Model # 5HS-7010 |
| Type 3: | Standpipe stainless steel overflow tube: 1-1/2” NPSM stainless steel drain; 1-1/2” NPSM brass locknut; 1-1/2” rubber washer TANDS Model # : 5CS-72I Overflow Tube TANDS Model # : 5HS-7001 Stainless Steel Drain TANDS Model # : 5GB-700 Brass Locknut TANDS Model # : 5FR-700 Rubber Washer |
| Type 4: | Standpipe stainless steel overflow tube: 1” NPSM drain; 1” NPSM brass locknut. TANDS Model # : 5CH-52H Overflow Tube TANDS Model # : 5An-50F Drain TANDS Model # : 5GB-5000 Brass Locknut |
| Type 5: | 1-1/2” NPSM stainless steel drain; 1-1/2” NPSM brass locknut; ½” rubber washer. TANDS Model # : 5HS-7001 Stainless Steel Drain TANDS Model # : 5GB-700 Brass Locknut TANDS Model # : 5FR-700 Rubber Washer |
| Type 6: | Drain: rotary waste valve with twist handle, 3-1/2” sink opening, 2” NPT male outlet AND 1-1/2” NPT male adapter AND 19” x 15” overflow tube with head assembly. 1-1/2” NPT waste valve with removable snap-in strainer. TANDS Model # B-3950-01 |
| Type 7: | Drain: rotary waste valve with twist handle, 3-1/2” sink opening, 2” NPT male outlet AND 1-1/2” NPT male adapter AND 19-1/2” x 15” overflow tube with head assembly. 1-1/2” NPT waste valve with removable snap-in strainer. TANDS Model # B-3950-01-SB |

- Quick Disconnect Valves

  - Water Lines

    - Flexible water hoses with quick disconnect for ice machines, coffee and tea brewers and any mobile equipment requiring water connections.  SwirlTM Water Supply Line shall be manufactured by Dormont Manufacturing Co. and be a coiled retractable polyurethane hose that is NSF certified with a maximum allowable temp of 160 degrees and maximum allowable pressure 100 PSIG.  Lead Free Brass Quick Disconnect fitting to be provided with 2-way shut off to stop water at supply side and prevent back flow from equipment when disconnected. Flexible water line must be of sufficient length to attach properly to the device and include all necessary fittings and related appurtenances required for the proper operation of the assembly.

- Water Filters

  - All ice machines, combi-ovens, coffee and tea makers or urns, carbonated beverage dispensers and steam equipment shall have a water filter of proper type as manufactured by OptiPure, or as required by the equipment manufacturer.

- VENTILATION WORK – N/A

- ELECTRICAL WORK

- For all fabricated equipment, furnish, install (and intertwine from load center) all outlets, switches, controls, conduit, service fittings and load centers as shown as specified for the specific fixture.  Load centers shall be complete with individual “visi-trip” circuit breakers for each device built into or forming an integral part of the unit.  Furnish to Division 26 a wiring schematic including circuit breaker diagram for load center.

- Insure that all equipment furnished under this contract shall be so wired, wound or constructed as to conform with the characteristics of electrical and other services at the premises.

- Appliances shall be new, of manufacturer’s current production and furnished complete with motors drive mechanism and other electrical equipment if and as applicable.  Wiring and connection diagrams shall be furnished with electricity operated machines and for all fabricated equipment.

- All conduit wiring shall be run concealed wherever possible.  Conduit shall be continuous from outlet to outlet and from outlet to load center circuit or pull boxes and shall center and be secured in such a manner that each system shall be electrically continuous throughout.  All conduits shall be thoroughly and substantially supported by accepted industry practices.

- Supply on each motor driven appliance or electrical heating unit, a suitable control switch or starter of proper type wherever such equipment is not so built.

- All plug-in equipment shall have plugs and cords furnished and installed.  Coordinate work with Division 26 so that the receptacles provided will match the specific plugs installed as part of the plug-in equipment.  any changes on cords and plugs required in the field due to lack of coordination between Division 26 and Kitchen Equipment Contractor shall be the latter’s responsibility.

- All surface mounted receptacles indicated for fabricated equipment are to have Component Hardware Model R58-1010 or equal aluminum box complete with satin finish stainless steel cover and receptacle as indicated below:

  - 2-pole, 3-wire grounding 20 amp; 125 V.  
Hubbell # 5352 or equal (NEMA 5-20R).

  - 2-pole, 3-wire grounding 20 amp; 250 V.  
Hubbell # 5461 or equal (NEMA 6-20R).

  - 2-pole, 3-wire grounding 30 amp; 250 V.  
Hubbell # 9330 or equal (NEMA -30R).

- All built-in receptacles indicated for fabricated equipment are to be 2” x 4” x 1-1/2” (50 mm x 100 mm x 38 mm) deep “Handy Box” tack welded to fixture and fitted with receptacle indicated above and satin finish stainless steel cover.  Splash mounted receptacles to be horizontal with all other vertical.

  - 30 AMP, 250 V receptacles require a 2-1/8” (54 mm) deep “Handy Box”.  If splash width to 2-1/2” (62 mm).

- All electrically heated, fabricated equipment shall be internally wired to a thermostatic control and an “on/off” red light indicator, both to be mounted in a terminal box with a removable access panel and located outside the heating area.  Wiring to be nickel-plated copper, properly insulated.

- Provide all incandescent/LED bulbs and fluorescent/LED tubes required for equipment under this section.

- FABRICATED EQUIPMENT – N/A

- MILLWORK EQUIPMENT – N/A

- STANDARD DETAILS – N/A

- COLD STORAGE ROOMS – N/A

- REMOTE REFRIGERATION SYSTEMS – N/A

- EXECUTION

- DELIVERY AND INSTALLATION

- Delivery

- The equipment shall be delivered and installed on schedule.  Coordinate all work with the General Contractor and other divisions as required.

- Extra charges resulting from special handling or shipment shall be paid by the Kitchen Equipment Contractor if insufficient time was allowed in placing factory orders to ensure normal shipment.

- The work shall be accomplished so as not to delay the project construction schedule, interfere or conflict with the work being performed by other contractors.  Work shall be coordinated and integrated to prevent conflict of work necessitating changes to work already completed.  Should conflicts occur, notify the Owner for his coordination in its resolution.

- Verify all field dimensions before fabrication.

- Include all alterations to walls, floors and ceiling necessary for work, except otherwise shown or specified, accomplished in a manner satisfactory to the Architect and the Designer.  Holes through structural beams shall be prohibited unless written approval has been granted by the Architect.

- Cut holes in equipment for pipes, drains, electric outlets, etc. as required for this installation.  Work shall conform to highest standards or workmanship and shall include welded sleeves, collars, ferrules or escutcheons.

- Repair all damage to the premises as a result of this installation.

- Remove daily all debris from the site related to this installation.

- Remove any plates, components or component covers installed at the factory before installing the FRP-X panels at cold storage rooms and reinstall them afterwards along with the items furnished loose for mounting on the exterior face of the wall panels.

- Space between all equipment to wall, ceiling, floors, masonry pads, and adjoining units not portable and with enclosed bodies shall be completely sealed against entrance of food particles or vermin by means of trim strips, welding, soldering or mastic.  Mastic shall be General Electric Silicone Construction Sealant Series SE1200 in appropriate color.

- Trade marks and names of fabricator shall not be fastened to any items without the written approval of Clevenger Associates.

- All items shall be installed plumb, square, level and in proper elevation, plane location and in alignment with other work.

- START-UP AND DEMONSTRATION

- All equipment under this section shall be cleaned and ready for operation at time building is turned over to the Owner.

- Provide a competent service representative to be present when installation is put into operation.  He shall lubricate and put into proper operation all equipment and instruct the Owner’s employees in the proper use and maintenance of all items in this contract and set up a maintenance schedule to be followed thereafter.  Three (3) copies of the schedule shall be provided before final acceptance of the installation.

- During start-up provide all required instruction for operation and maintenance of equipment, after one (1) year guarantee period.

- MAINTENANCE SCHEDULE

- Provide operation and service inspections every ninety (90) days during the warranty period.  Final inspection shall be performed thirty (30) days before warranty expiration.  Any service or repair requirements shall be performed before the end of the warranty period.

- Copies of all warranty service calls and inspection reports shall be mailed to the Owner and Building Operations Engineer.

- The Owner may call an outside company at the expense of the Kitchen Equipment Contractor, if the Kitchen Equipment Contractor does not arrive within a reasonable amount of the called in response to an emergency call.

- EQUIPMENT

- REGULAR MANUFACTURED EQUIPMENT

A.	Provide equipment with standard finishes and accessories unless specifically deleted or superseded by the Contract Documents.

- FABRICATED EQUIPMENT – N/A

- FOODSERVICE EQUIPMENT SCHEDULE

| Item No. | Qty | Description |
| --- | --- | --- |
| 1 | 1 | DISPLAY FREEZER |
| 2 | 1 | DISPLAY REFRIGERATOR |
| 3 | 1 | WARMING KETTLE |
| 4 | 1 | UNDERCOUNTER DISPLAY REFRIGERATOR |
| 5 | 1 | REFRIGERATED COLD WELL (DROP-IN) |
| 6 | 2 | MICROWAVE OVEN |
| 7 | 1 | WAFFLE IRON (OPERATOR FURNISHED) |
| 8 | - | SPARE NUMBER |
| 9 | 1 | JUICE DISPENSER (OPERATOR FURNISHED) |
| 10 | 1 | CONVEYOR TOASTER |
| 11 | 1 | BATTER MIX STARTER KIT (OPERATOR FURNISHED) |
| 12 | 1 | WAFFLE IRON (FUTURE) (OPERATOR FURNISHED) |
| 13 | 1 | HAND SINK WITH SOAP AND TOWEL DISPENSER |
| 14 | 1 | COFFEE MAKER (OPERATOR FURNISHED) |
| 15 | 1 | WALL SHELF (HIGH MOUNT) |
| 16 | 1 | WORKTABLE |
| 17 | 2 | WALL SHELF |
| 18 | 1 | MOBILE TABLE |
| 19 | 1 | MOBILE STORAGE SHELVING |
| 20 | - | SPARE NUMBER |
| 21 | - | SPARE NUMBER |
| 22 | - | SPARE NUMBER |
| 23 | 1 | MOBILE REACH-IN REFRIGERATOR |
| 24 | 1 | MOBILE REACH-IN FREEZER |
| 25 | 1 | UTENSIL RACK |
| 26 | 1 | DISH/POT WASHING SINKS |
| 27 | 3 | WALL SHELF (HIGH MOUNT) |
| 28 | 1 | PRE-RINSE WITH FAUCET |
| 29 | 1 | RACK SHELF (WALL MOUNT) |
| 30 | 1 | UNDERCOUNTER DISHMACHINE (HIGH TEMP) |

- FOODSERVICE EQUIPMENT ITEMIZED

ITEM 1:	DISPLAY FREEZER

Quantity:	One (1)

Manufacturer:	True

Model:	GDM-23F-LD

          - Unit to be complete with all standard components and/or accessories including the following:

  - Freezer shall have glass door hinged as indicated on the drawings, provided with factory installed lock.

  - Cord and plug set.

  - Provide two (2) additional wire shelves.

  - Five (5) year extended compressor warranty and two (2) year labor warranty.

  - Verify sign panel with Owner.

ITEM 2:	DISPLAY REFRIGERATOR

Quantity:	One (1)

Manufacturer:	True

Model:	GDM-23-LD

          - Unit to be complete with all standard components and/or accessories including the following:

            - Refrigerator shall have glass door hinged as indicated on the drawings, provided with factory installed lock.

            - Cord and plug set.

            - Provide two (2) additional wire shelves.

            - Five (5) year extended compressor warranty and two (2) year labor warranty.

            - Verify sign panel with Owner.

ITEM 3:	WARMING KETTLE

Quantity:	One (1)

Manufacturer:	Vollrath

Model:	72017-7

- Unit to be complete with all standard components and/or accessories including the following

  - Kool-touch hinged cover

ITEM 4:	UNDERCOUNTER DISPLAY REFRIGERATOR

Quantity:	One (1)

Manufacturer:	True

Model:	TUC-27G-LP

          - Unit to be complete with all standard components and/or accessories including the following:

            - Refrigerator shall have glass door hinged as indicated on the drawings, provided with factory installed lock.

            - Cord and plug set.

            - Provide exterior thermometer.

            - Provide one (1) additional wire shelf.

            - Five (5) year extended compressor warranty and two (2) year labor warranty.

ITEM 5:	REFRIGERATED COLD WELL (DROP-IN)

Quantity:	One (1)

Manufacturer:	APW Wyott

Model:	CW-1

          - Unit to be complete with all standard components and/or accessories including the following:

- Unit to be installed in Millwork Counter as shown on drawings, coordinate with Millwork Fabricator.

- Five (5) year extended compressor warranty and two (2) year labor warranty.

ITEM 6:	MICROWAVE OVEN

Quantity:	Two (2)

Manufacturer:	Amana

Model:	HDC12A2

          - Unit to be complete with all standard components and/or accessories including the following:

            - Braille touch pads.

ITEM 7:	WAFFLE IRON (OPERATOR FURNISHED)

Quantity:	One (1)

          - This item is not included in this Part of the Work.  Kitchen Equipment Contractor shall schedule and coordinate his work with that of the other trades to expedite the job progress.

ITEM 8:	SPARE NUMBER

ITEM 9:	JUICE DISPENSER (OPERATOR FURNISHED)

Quantity:	One (1)

          - This item is not included in this Part of the Work.  Kitchen Equipment Contractor shall schedule and coordinate his work with that of the other trades to expedite the job progress.

ITEM 10:	CONVEYOR TOASTER

Quantity:	One (1)

Manufacturer:	APW Wyott

Model:	ECO 4000-350L-HILTON

          - Unit to be complete with all standard components and/or accessories.

ITEM 11:	BATTER MIX STARTER KIT (OPERATOR FURNISHED)

Quantity:	One (1)

          - This item is not included in this Part of the Work.  Kitchen Equipment Contractor shall schedule and coordinate his work with that of the other trades to expedite the job progress.

ITEM 12:	WAFFLE IRON (FUTURE) (OPERATOR FURNISHED)

Quantity:	One (1)

          - This item is not included in this Part of the Work.  Kitchen Equipment Contractor shall schedule and coordinate his work with that of the other trades to expedite the job progress.

ITEM 13:	HAND SINK WITH SOAP AND TOWEL DISPENSER

Quantity:	One (1)

Manufacturer:	Advance Tabco

Model:	7-PS-87

          - Unit to be complete with all standard components and/or accessories.

ITEM 14:	COFFEE MAKER (OPERATOR FURNISHED)

Quantity:	One (1)

          - This item is not included in this Part of the Work.  Kitchen Equipment Contractor shall schedule and coordinate his work with that of the other trades to expedite the job progress.

ITEM 15:	WALL SHELF (HIGH MOUNT)

Quantity:	One (1)

Manufacturer:	Metro

Model:	2436NC/1WD24C

          - Unit to be complete with all standard components and/or accessories including the following:

            - Two (2) Model 1WD24C Direct Wall Mount Shelf Supports.

            - One (1) Model 2436NC Wire Shelves.

            - Install shelf surface at 7’-6” A.F.F.

ITEM 16:	WORKTABLE

Quantity:	One (1)

Manufacturer:	Advance Tabco

Model:	KMS-304

          - Unit to be complete with all standard components and/or accessories.

            - Worktable shall be shaped and sized as detailed on the drawings approximately 4’-0” x 2’-7” x 3’-0” high.

            - Provide back and left side splashes as shown on drawings.

            - Full length bottom shelf below as shown on drawings

            - See Elevation “C” Sheet FS-5 for additional information.

ITEM 17:	WALL SHELF

Quantity:	Two (2)

Manufacturer:	Advance Tabco

Model:	WS-12-24

          - Unit to be complete with all standard components and/or accessories including the following:

            - Provide one (1) at 5’-4” above finish floor and one (1) at 6’-4” above finish floor.

            - See Elevation “C” Sheet FS-5 for additional information.

ITEM 18:	MOBILE TABLE

Quantity:	One (1)

Manufacturer:	Advance Tabco

Model:	MS-245

          - Unit to be complete with all standard components and/or accessories including the following:

            - Worktable shall be shaped and sized as detailed on the drawings approximately 5’-0” x 2’-0” x 3’-0” high.

            - Full length bottom shelf below as shown on drawings.

            - Provide set of four (4) casters, two (2) with brakes.

ITEM 19:	MOBILE STORAGE SHELVING

Quantity:	One (1)

Manufacturer:	Metro

Model:	Metromax Q

          - Unit to be complete with all standard components and/or accessories including the following:

            - Shelving shall be sized as indicated on the plan.

            - Section shall consist of four (4) posts and five (5) shelves.

            - Posts shall be MQ86UPE.

            - Bottom shelf shall be set 10” above finish floor line and the four (4) remaining shelves equally spaced.

            - Provide two (2) Model 5PCXM and two (2) Model 5PCBXM casters per unit.

ITEM 20:	SPARE NUMBER

ITEM 21:	SPARE NUMBER

ITEM 22:	SPARE NUMBER

ITEM 23:	MOBILE REACH-IN REFRIGERATOR

Quantity:	One (1)

Manufacturer:	True

Model:	T-49-HC

          - Unit to be complete with all standard components and/or accessories including the following:

            - Refrigerator shall have doors hinged as indicated on the drawings, provided with factory installed locks.

            - Cord and plug set.

            - Provide exterior thermometer.

            - Provide four (4) additional wire shelves.

            - Provide set of four (4) casters, two (2) with brakes.

            - Five (5) year extended compressor warranty and two (2) year labor warranty.

ITEM 24:	MOBILE REACH-IN FREEZER

Quantity:	One (1)

Manufacturer:	True

Model:	T-49F-HC

          - Unit to be complete with all standard components  and/or accessories including the following:

            - Freezer shall have doors hinged as indicated on the drawings, provided with factory installed locks.

            - Cord and plug set.

            - Provide exterior thermometer.

            - Provide four (4) additional wire shelves.

            - Provide set of four (4) casters, two (2) with brakes.

            - Five (5) year extended compressor warranty and two (2) year labor warranty.

ITEM 25:	UTENSIL RACK

Quantity:	One (1)

Manufacturer:	Advance Tabco

Model:	PS-12-60

          - Unit to be complete with all standard components and/or accessories including the following:

            - Install shelf surface at 6’-6” A.F.F.

            - See Elevation “D” Sheet FS-5 for additional information.

ITEM 26:	DISH/POT WASHING SINKS

Quantity:	One (1)

Manufacturer:	Advance Tabco

Model:	94-23-60-24RL (CUSTOM)

          - Unit to be complete with all standard components and/or accessories including the following:

            - Dishtable shall be shaped and sized as detailed on the drawings approximately 9’-7” x 2’-3” x 3’-1” high.

            - Provide one (1) T & S Model B-0275 Faucet, location centered between right and middle sink tubs.

            - Set of 8” on center faucet holes centered over left hand sink to accommodate Item 28, Pre-Rinse with Faucet.

            - Provide three (3) T & S Model B-3950-01-SB Rotary Waste Drains with overflows.

            - Space below right side drainboard to accommodate Item 30, Undercounter Dishmachine (High Temp).

            - Provide back and left side splashes as shown on drawings.

            - See Elevation “D” Sheet FS-5 for additional information.

ITEM 27:	WALL SHELF (HIGH MOUNT)

Quantity:	Three (3)

Manufacturer:	Metro

Model:	2436NC/1WD24C/2WD24C

          - Unit to be complete with all standard components and/or accessories including the following:

            - Two (2) Model 1WD24C Direct Wall Mount Shelf Supports.

            - Two (2) Model 2WD24C Direct Wall Mount Shelf Supports.

            - Three (3) Model 2436NC Wire Shelves.

            - Install shelf surface at 7’-6” A.F.F.

ITEM 28:	PRE-RINSE WITH FAUCET

Quantity:	One (1)

Manufacturer:	T & S Brass

Model:	B-2278-A12-EL

          - Unit to be complete with all standard components and/or accessories including the following:

            - Install at left hand sink of Item 26, Dish/Pot Washing Sinks.

ITEM 29:	RACK SHELF (WALL MOUNT)

Quantity:	One (1)

Manufacturer:	Advance Tabco

Model:	DT-6R-11

          - Unit to be complete with all standard components and/or accessories including the following:

            - Install bottom of shelf at 4’-6” A.F.F.

            - See Elevation “D” Sheet FS-5 for additional information.

ITEM 30:	UNDERCOUNTER DISHMACHINE (HIGH TEMP)

Quantity:	One (1)

Manufacturer:	Champion Industries

Model:	UH-100B

          - Unit to be complete with all standard components and/or accessories including the following:

            - Provide 70F° Rise built in booster heater.

            - Electric:  208/60/3 Phase.

- FOODSERVICE STANDARD DETAILS – N/A

END OF SECTION 11 40 00